FUNDAMENTALS OF COMPOSITE MATERIALS  |  21MEO101T

WaveSense-CFRP

NOVELTY-FOCUSED TECHNICAL PROJECT DOSSIER

Temperature-Robust, Sensor-Efficient and Reliability-Aware Ultrasonic Guided-Wave Damage Assessment of Carbon Fiber-Reinforced Polymer Laminates

Proposed technical contribution: temperature-stability-aware sensor-path selection combined with missing-measurement robustness and calibrated, abstention-capable diagnosis.

SUBMISSION INFORMATION

| Field | Details |
| --- | --- |
| Institution | SRM Institute of Science and Technology |
| Course / code | Fundamentals of Composite Materials / 21MEO101T |
| Department offering course | Mechanical Engineering |
| Student / registration no. | ________________________________________ |
| Faculty guide / section | ________________________________________ |
| Proposed project format | Software-only research and engineering prototype |
| Proposal version / date | Version 2.0 / 14 October 2026 |

STATUS: Proposed research and implementation plan. No model has yet been trained, and no experimental performance or novelty breakthrough is claimed.

# Executive decision brief

This proposal upgrades the original WaveSense-CFRP concept from standard ultrasonic damage classification to a reliability-focused inspection framework. The project is fully digital: real signals were acquired by external laboratories, while our team will create original preprocessing, path-selection, analysis, model-evaluation and reporting software.

## Proposed core contribution

Temperature-aware path ranking: identify transmitter–receiver paths and excitation frequencies with strong reference-damage discriminability and comparatively low healthy-state thermal variability.

Sensor efficiency and fault resilience: quantify classification performance using limited path budgets and when an entire transducer or selected measurements are unavailable.

Reliability-aware decisions: calibrate model scores and issue Healthy / Reference damage / Inconclusive decisions rather than forcing unreliable classifications.

Leakage-resistant experimental design: test held-out temperature intervals and, where feasible, unseen reference-damage positions with selection performed only on training data.

## Faculty-ready abstract

Ultrasonic guided waves are widely investigated for condition monitoring of fiber-reinforced composite structures. Their propagation is sensitive not only to changes associated with structural discontinuities but also to environmental temperature, creating challenges for reliable damage detection. WaveSense-CFRP proposes a reproducible, software-only inspection framework using the publicly available Open Guided Waves (OGW) Dataset #2, which records multi-path ultrasonic responses of a carbon fiber-reinforced polymer (CFRP) laminate across controlled temperatures and reversible reference-damage conditions. The proposed method ranks measurement paths by joint damage sensitivity and healthy-state thermal stability, selects informative subsets under an explicit measurement budget, and evaluates supervised classifiers alongside signal-processing baselines. Robustness is assessed through held-out-temperature evaluation and simulated unavailable measurements, including transducer-level failures. Confidence calibration and an abstention mechanism identify inconclusive cases. The work will deliver an independently implemented algorithm, a transparent evaluation protocol, visual analytics and an interactive inspection dashboard. The aim is to determine whether temperature-aware sensor-path selection can improve the reliability-versus-measurement-efficiency trade-off; any improvement will be reported only after experimental validation.

## Proposed outcomes - not promises

| Research output | Evidence required |
| --- | --- |
| Temperature-aware ranking method | Explicit algorithm, reproducible code, train-only parameter selection |
| Comparison with existing baselines | Held-out metrics, ablations, uncertainty intervals |
| Path efficiency report | F1/recall/false-alarm rate against number of paths and unique sensors |
| Reliability module | Calibration plots, Brier score, risk–coverage trade-off |
| Digital engineering application | Live re-analysis of labeled public records and downloadable report |

# 01  Academic context and course alignment

The supplied syllabus identifies the open-elective course 21MEO101T, “Fundamentals of Composite Materials,” with learning outcomes covering composite constituents, laminate mechanics, composite processing, testing methods and advanced composite applications. Unit 4 explicitly covers non-destructive inspection using ultrasonics, and Unit 2 addresses laminate behavior and damage mechanisms. [S1]

| Unit | Relevance to WaveSense-CFRP |
| --- | --- |
| 1 - Introduction | CFRP constituents, reinforcement architecture, matrix and interfaces. |
| 2 - Mechanical analysis | Anisotropic laminated response; damage-related changes in mechanical wave propagation. |
| 3 - Composite processing | CFRP plate fabrication, laminate integrity and relevance of manufacturing quality. |
| 4 - Performance / quality | Direct connection to ultrasonic non-destructive testing and inspection methods. |
| 5 - Advanced composites | Advanced fiber-reinforced materials and condition-monitoring applications. |

# 02  Problem statement and research gap

A conventional classifier trained on ultrasonic signals under one set of conditions may exploit differences that are unrelated to damage, including temperature and recording-session artifacts. Using every path raises data processing demands and may over-represent redundant information. Missing transducer recordings can also degrade a system that assumes a complete sensor network. Furthermore, a forced binary prediction is not appropriate when confidence is low or the observation is outside conditions learned from training data.

## Existing work and defensible contribution

The OGW Dataset #2 source paper already demonstrates temperature-sensitive signals and reports baseline-selection, PCA/ICA and adaptive-reference approaches. Related literature includes temperature-compensation research and uncertainty assessment for guided-wave diagnosis. Therefore, none of “ML detection,” “temperature compensation,” “sensor selection,” or “uncertainty” is individually claimed as a first-time invention. [R1, R3, R4]

CLAIM OF NOVELTY: A project-specific, independently implemented and ablated combination of temperature-stability-aware path ranking, sensor-failure stress testing and abstention-calibrated classification. Its incremental research value must be established by comparison, not assumed.

## Research questions

RQ1. Does temperature-stability-aware path selection outperform random and conventional path selection at the same measurement budget on held-out temperature conditions?

RQ2. How does performance change when selected recording paths or all measurements involving one transducer become unavailable?

RQ3. Does confidence calibration with an inconclusive option reduce error among issued decisions, and at what coverage cost?

RQ4. How well does the method generalize to a reference-damage location that was not used for model selection or training?

## Working hypotheses

H1: The proposed path ranking will improve the detection-versus-path-budget trade-off under selected held-out temperatures. H2: Robust training or path substitution will reduce degradation under simulated missing measurements. H3: An abstention threshold chosen solely on validation data will reduce error among accepted predictions while reducing coverage. These are testable hypotheses, not claimed results.

# 03  Technical background

Ultrasonic guided waves propagate within the boundaries of thin plate-like structures. In layered CFRP, the behavior is influenced by laminate architecture, anisotropy, frequency, temperature and geometry. A transmitter launches a tone burst; receivers observe complex waveforms containing multiple arrivals, scattered components and reflections. Damage proxies can alter amplitude, phase and time-of-flight, but temperature can change these quantities without any defect. [R1]

Figure 1. Conceptual sensor-path subset on a plate. This is a schematic created for the proposal; it does not reproduce the publisher’s transducer coordinates.

## Interpretation rules

A detected signature is evidence of a changed guided-wave response under the modeled conditions, not proof of a specific internal failure mechanism.

The OGW reference damage consists of a surface-mounted aluminum disc attached using tacky tape. It is reversible and must not be described as an experimentally confirmed internal delamination. [R1]

A model validated on one specimen and controlled reference defects cannot be asserted to work on arbitrary real-world CFRP structures.

# 04  Dataset provenance and access

## Primary dataset: Open Guided Waves (OGW) Dataset #2

Source: Moll, Kexel, Pötzsch, Rennoch and Herrmann, Scientific Data 6, 191 (2019), “Temperature affected guided wave propagation in a composite plate complementing the Open Guided Waves Platform.” DOI: 10.1038/s41597-019-0208-1. Archive collection DOI: 10.6084/m9.figshare.c.4488089.v1. The official OGW site links to the publisher-hosted files. [R1, D1]

| Published property | Description |
| --- | --- |
| Specimen | Quasi-isotropic HexPly CFRP laminate; 500 x 500 mm; 2 mm thick. |
| Laminate layup | [45/0/-45/90/-45/0/45/90]s, as reported by data authors. |
| Transducers | 12 integrated piezoelectric discs; round-robin transmitter–receiver measurements. |
| Excitation | 5-cycle Hann-windowed tone burst; 40–260 kHz at 20 kHz intervals. |
| Temperature | 20–60 degrees C; 0.5-degree increments during controlled chamber cycles. |
| Healthy/reference states | Two intact temperature cycles; one cycle for each of four disc positions. |
| Damage positions | D04, D12, D16, D24; removable surface-mounted aluminum-disc proxy. |
| Raw data | HDF5 (.h5) inside timestamped folders; filenames identify carrier frequency. |
| Research use | Re-analysis and algorithm comparison with attribution to data creators. |

CRITICAL DATA LIMITATION: Each damaged configuration belongs to a separate recorded cycle, and there is one principal specimen. Condition, temperature trajectory, day/cycle and location may be partially confounded. Treat record-level random splits as invalid evidence of real-world generalization.

## Five published folder categories

OGW_CFRP_Temperature_udam - intact baseline

OGW_CFRP_Temperature_dam_D04 - proxy at D04

OGW_CFRP_Temperature_dam_D12 - proxy at D12

OGW_CFRP_Temperature_dam_D16 - proxy at D16

OGW_CFRP_Temperature_dam_D24 - proxy at D24

## Secondary sources and use constraints

| Dataset | Appropriate use | Decision |
| --- | --- | --- |
| OGW Dataset #1 | Additional constant-temperature / artificial-reference-damage analysis. | Optional transfer check; do not pool naively. |
| NASA CFRP composites | Different fatigue and sensor experiment; possible later research extension. | Out of primary course scope. |
| Other experimental delamination benchmarks | Test whether methods transfer to real/internal-like damage. | Requires separate domain and label audit. |

## Dataset access checklist before modeling

Retrieve the official dataset catalogue and source article; store DOI, version, download date and file checksums.

Download a manageable subset and verify file readability, time axis, sampling rate, frequency tags and 12-channel relationships.

Count unique acquisitions, temperature-cycle identifiers, source files, transmitters, receivers and class labels.

Measure storage/memory requirements locally before deciding whether to use all frequencies and sensor paths.

Freeze the metadata manifest and split protocol before any supervised feature ranking.

# 05  Proposed data model and signal preprocessing

## One canonical acquisition record

Each analysis unit will be linked to a complete measurement acquisition rather than treating each waveform from that acquisition as an independent statistical observation. Proposed metadata keys are listed below; exact native HDF5 fields will be reconciled after inspection.

| Field | Purpose |
| --- | --- |
| source_file / source_hash | Reproducible provenance and integrity |
| condition / damage_position | Intact or D04/D12/D16/D24 reference-damage label |
| temperature_C / cycle_id / timestamp | Environmental condition and leakage control |
| frequency_kHz / transmitter / receiver | Frequency-specific measurement path ID |
| sampling_interval / waveform | Time-base interpretation and raw signal |
| acquisition_group | Split identifier linking correlated recordings |

## Preprocessing workflow

Ingest HDF5 data using h5py, validate metadata, and preserve read-only raw signals.

Remove malformed traces; verify finite sample values, channel lengths and monotonic time axes.

Remove or separately model the excitation/crosstalk region using a pre-declared physically justified time window.

Apply frequency-aware filtering and trace normalization using training-only parameters; examine effect of filtering by ablation.

Align signals and choose healthy references only from approved training/calibration pools, never by looking up an unknown test label.

Extract time-, frequency- and time-frequency-domain features; cache them with the preprocessing settings and software version.

## Candidate features

| Category | Feature examples / interpretation |
| --- | --- |
| Time domain | RMS, peak amplitude, energy, envelope, crest factor |
| Similarity | Normalized cross-correlation, residual energy, relative baseline change |
| Timing | Time-of-flight lag, peak arrival shift after alignment |
| Frequency | FFT-band energy, centroid, spectral change |
| Time-frequency | Wavelet-energy descriptors where computationally justified |

A preliminary path-specific residual-energy index can be written as DI = ||x_test - x_ref||_2 / (||x_ref||_2 + epsilon). The operation presumes compatible scaling and time alignment. It is a change indicator, not a direct estimate of crack size or severity.

# 06  Proposed novelty algorithm: TASP

## Temperature-Aware Sensor-Path Prioritization (TASP)

We propose a transparent, train-derived path-and-frequency ranking score. For each candidate path p and frequency f, D(p,f) quantifies reference-damage discriminability using training-only labels. V(p,f) quantifies variability of the corresponding healthy features across training temperatures. An exploratory score is:

Q(p,f) = D(p,f) / [V(p,f) + epsilon]. Final ranking will also consider path redundancy, path reliability and possibly the number of unique transducers required.

Possible definitions include a robust median-separation effect size for D and a temperature-conditioned median absolute deviation for V. Because the ratio can reward unstable low-variance estimates, all components will be normalized, uncertainty-checked, and ablated; simple alternatives (e.g. additive weighted ranking) will be tested.

## Greedy constrained selection

Compute candidate features and path rankings using training acquisitions only.

Select the highest-scoring candidate and penalize redundant correlation with already selected paths.

Repeat until the chosen path budget K or unique-transducer budget is reached.

Use a nested validation scheme to tune K, weighting coefficients, and model hyperparameters.

Freeze the selected path list before evaluating each held-out fold; rebuild from scratch for the next fold.

## Mandatory comparisons (to establish incremental value)

| Baseline | Purpose |
| --- | --- |
| All paths, fixed features | Upper-resource reference; may not be best under shift |
| Random K paths (many seeds) | Checks improvement beyond path-number reduction |
| Damage-only discriminability | Tests contribution of thermal stability term |
| Thermal-stability-only ranking | Tests whether stability alone is sufficient |
| Conventional feature selection | Compare to e.g. mutual information / RFE, if time permits |
| TASP without redundancy penalty | Ablates non-redundancy term |

The TASP name is an internal project label, not a claim that the mathematical idea is unprecedented. Publication-level novelty requires a deeper prior-art search and experimental superiority.

# 07  Missing-measurement and sensor-failure robustness

Two levels of simulated failure will be evaluated on recorded data. (A) Path dropout removes random or strategically selected path/frequency records. (B) Transducer outage removes every path where a selected transducer acts as transmitter or receiver. This distinction is essential: fewer paths do not necessarily mean fewer physical sensors.

## Scenarios

| Scenario | Method | Question answered |
| --- | --- | --- |
| Random path loss | Mask 10%, 25%, 50% of selected paths | Graceful degradation under sporadic missing records? |
| Single-sensor outage | Remove all paths involving one transducer | Tolerance to one unavailable physical transducer? |
| Worst-case path loss | Remove top-ranked paths first | Dependence on narrow critical paths? |
| Train-time path masking | Apply masks only inside training partitions | Does mask-aware training improve fault tolerance? |

Missing paths will be represented explicitly as unavailable rather than silently replaced with fabricated healthy measurements. Strategies may include mask features, aggregate statistics over available paths, and a fallback selector that uses pre-ranked substitutes. All substitutes are chosen without access to test labels.

# 08  Reliability-aware decision logic

## Baseline model family

The primary classifier will use interpretable features with Logistic Regression and Random Forest or SVM benchmarks. One-class healthy-reference anomaly detection may be included as a robustness baseline, particularly for unseen damage positions. Large neural networks are not required for the core project.

## Calibration and selective prediction

Classification probabilities will be checked using Brier score, reliability diagrams, and calibration error. If supported by adequate validation data, calibrated probabilities will be obtained using a reserved calibration fold. Thresholds for issuing an automatic decision will be fixed from calibration data only; uncertain outputs become Inconclusive, without using ground truth from the inspected case.

| Software output | Decision interpretation |
| --- | --- |
| No reference damage detected | Selected test signal is classified as healthy within validated domain. |
| Reference damage indicated | Signal resembles experimental reversible reference-damage conditions. |
| Inconclusive / review required | Uncertainty, missing paths or out-of-domain conditions prevent a reliable decision. |

## Proposed guardrails

Display temperature, frequency, available path count, and test-domain coverage alongside the result.

Warn when the temperature is outside the validated range or when an unknown frequency/sensor ID is provided.

Do not equate an SVM probability or model confidence with physical certainty of damage.

Use selective risk-versus-coverage evaluation: a lower error rate is meaningful only with its accompanying abstention rate.

# 09  Evaluation protocol and reproducible experiments

## Splitting policy and the confounding problem

All paths and frequencies originating from one underlying acquisition must belong to the same partition. Consecutive temperatures within a recording cycle are correlated, so simply shuffling individual waveforms is prohibited. Temperature-band holdout (e.g. contiguous ranges) and cycle-conscious analysis are preferred. With only one damaged cycle per reference location, a strict across-cycle test may be inseparable from a change in damage position; the report will explicitly separate what is identifiable from what is confounded.

Leave-one-damage-location-out analysis is planned only as a stress test: supervised ranking and classifier fitting must exclude that position entirely. Success in this setting cannot prove localization performance on actual delaminations or independent specimens.

## Experiment matrix

| ID | Experiment | Primary output |
| --- | --- | --- |
| E0 | Manifest, provenance and class-balance audit | Data audit table; reproducible partitions |
| E1 | All-path signal index and classical ML baselines | F1, recall, false-alarm rate, ROC-AUC where valid |
| E2 | Blocked temperature generalization | Performance vs. held-out temperature band |
| E3 | TASP vs random / damage-only / stability-only | Performance vs. number of paths and unique sensors |
| E4 | Dropout and simulated single-sensor outage | Robustness curves and failure-case analysis |
| E5 | Calibration and abstention threshold | Brier, calibration diagram, risk–coverage |
| E6 | Leave-one-reference-damage-position-out | Exploratory transfer result; cautious interpretation |

## Reporting metrics

Classification: balanced accuracy, macro F1, reference-damage recall, healthy false-positive rate, confusion matrices and ROC-AUC/PR-AUC where applicable. Calibration: Brier score, calibration curve and expected calibration error with justified binning. Sensor/path efficiency: number of selected paths, unique transducers, feature extraction time, model inference time. Reliability: selective risk, coverage and robustness degradation. Uncertainty: bootstrap confidence intervals resampled at the appropriate acquisition/cycle group level; dependence limitations will be disclosed.

## What success looks like

Mandatory project success means reproducing the experimental protocol, completing baseline comparisons, showing transparent plots and delivering a functional application even if TASP does not improve accuracy. A scientific improvement claim requires measured benefit on held-out groups at equal path/sensor budget with uncertainty and counterexamples reported. No numeric target or expected accuracy is fabricated at proposal stage.

# 10  System architecture and demonstration

Figure 2. Proposed software pipeline. All methods, comparisons and outcomes shown are planned; this is not a results figure.

## Proposed software stack

| Layer | Selection |
| --- | --- |
| Data loading | Python, h5py, NumPy, pandas |
| Signal processing | SciPy (filters, FFT, cross-correlation), optional PyWavelets |
| Models and calibration | scikit-learn |
| Ranking and optimization | NumPy / SciPy + reproducible custom TASP module |
| Visualization | Matplotlib / Plotly |
| User interface | Streamlit running locally or via an available free-hosting option |
| Version control and testing | Git, pytest, configuration files, fixed random seeds |

## Dashboard modules

Dataset explorer: browse scenario, temperature, frequency, transmitter and receiver.

Signal diagnostics: healthy/reference waveform overlay, FFT, windowed residuals, damage-index plots.

Sensor efficiency: selected-path map, path-frequency score table and budget-performance plots.

Fault simulator: disable particular paths or an entire transducer, rerun diagnosis and track coverage.

Reliability tab: calibrated probability, confidence warning, reason for abstention and diagnostic report export.

Research tab: compare models using locked test partitions and display reproducibility metadata.

## Sample faculty demonstration

Choose a held-out public acquisition and visualize its received guided-wave traces.

Compare ordinary all-path inference with a TASP-ranked reduced-path configuration.

Change the virtual availability of transducers and observe failure or abstention behavior.

Display the decision, calibration status and validation-domain notes.

Export an inspection-style PDF/CSV report with model and dataset provenance.

# 11  Work plan, milestones, and dependencies

| Week | Scope | Acceptance checkpoint |
| --- | --- | --- |
| 1 | Acquire/audit OGW #2; build HDF5 reader, metadata manifest and splits. | Integrity audit plus first valid waveform plots. |
| 2 | DSP pipeline and baseline damage indices/classifiers. | Reproducible baseline metrics without leakage. |
| 3 | Implement TASP ranking and path-budget studies. | Ablations and budget plots on reserved validation. |
| 4 | Develop path dropout, transducer outage and mask-aware strategies. | Failure stress-test matrix and curves. |
| 5 | Calibrate scores; selective prediction and grouped robustness tests. | Calibration/risk–coverage and held-out evaluation. |
| 6 | Build dashboard, interpret findings, package code and final report. | Live reproducible demonstration and dossier-to-results traceability. |

## Baseline and stretch scope

| Committed minimum | Conditional advanced extension |
| --- | --- |
| Temperature-band evaluation, TASP ranking, path budgets, single-transducer dropout, selective prediction, Streamlit dashboard. | Leave-one-location-out tests, anomaly-detection benchmark, additional OGW dataset transfer, expanded uncertainty analyses. |

## Dependencies

Existing laptop and stable internet for dataset download; Python 3.11+ environment; adequate local disk for a selected OGW subset (full archive size to be confirmed); no sensors, test chamber, composite sample, Arduino, DAQ, finite-element software license or paid cloud GPU required. A GPU is optional, not part of core scope.

# 12  Budget and feasibility

| Category | Expected outlay | Notes |
| --- | --- | --- |
| Original experimental data | INR 0 | Publicly accessible OGW dataset; download/connection costs not estimated. |
| Python and numerical libraries | INR 0 | Open-source stack; verify package licenses before redistribution. |
| Sensors, specimen, laboratory equipment | INR 0 | Existing experimental records only. |
| Computing | INR 0 incremental | Assumes access to an existing capable laptop. |
| Report and demonstration software | INR 0 | Local Streamlit and standard office exports. |

Estimated direct project materials/software cost: INR 0 under existing-laptop and internet assumptions. Disk, data transfer, electricity and optional printing are not quantified.

# 13  Risks, limitations and mitigations

| Risk / limitation | Planned control |
| --- | --- |
| Large/complex HDF5 archive | Start with 1-2 frequencies; verify file schema and use chunked loading/caching. |
| Cycle/session confounding | Grouped acquisition splits, restricted conclusions, explicit experimental caveats. |
| Proxy defect is not delamination | Use “reference damage” in UI/report; no internal-delamination claims. |
| Insufficient independent data for calibration | Reserve calibration set, use simpler methods, show wide uncertainty; disable unsupported outputs. |
| Temperature-selection overfitting | Nested grouped validation; rank paths within training folds only. |
| Robustness under unseen conditions not established | Treat OOD alert as warning/diagnostic, not proof of robustness. |
| TASP fails to beat standard methods | Publish negative results, ablations and cost/error trade-offs; retain engineering contribution. |
| Overextended project scope | Lock baseline + core E0-E5; delay localization and extra datasets. |

# 14  Deliverables and faculty evaluation plan

## Submission package

A version-controlled Python project with documented environment setup and preprocessing scripts.

Immutable dataset manifest and checksum list; no redistribution of raw files unless permitted.

Implementation of TASP ranking, conventional baselines, robustness simulation and confidence calibration.

Reproducible experiment configuration and separated train/validation/test acquisition-group IDs.

Interactive WaveSense-CFRP dashboard with inspect/compare/failure/calibration views.

Final report with tables, figures, all negative or positive results, limitations, acknowledgments and citations.

Faculty presentation with live demo and one-page experimental summary.

## Faculty acceptance checklist

☐  Research question and constraints approved

☐  OGW Dataset #2 access/schema verified

☐  No ground-truth leakage in any model-selection step

☐  TASP compared against random and conventional ranking

☐  Temperature band and transducer outage evaluated

☐  Calibrated / inconclusive output documented

☐  Results separated from proposed capabilities

☐  Original data creators and prior art cited

## Project approval

| Faculty guide | Student / team representative |
| --- | --- |
| Name: __________________________ | Name: __________________________ |
| Signature: _______________________ | Signature: _______________________ |
| Date: ___________________________ | Date: ___________________________ |

# Appendix A  Implementation structure and reproducibility

Recommended repository layout (adaptable after HDF5 schema inspection):

•  data_manifest/ - provenance, checksums, split IDs and dataset notes

•  src/io/ - HDF5 ingestion and group metadata validation

•  src/dsp/ - filtering, baseline construction and engineered features

•  src/tasp/ - train-only temperature-aware path ranking

•  src/models/ - baselines, estimators, confidence calibration

•  src/stress/ - path dropout, sensor-outage and blocked evaluation

•  app/ - Streamlit visualization, classification and report view

•  experiments/ - fixed YAML configurations, scripts and saved metrics

•  tests/ - metadata, no-leakage and dropout behavior tests

## Data-leakage prevention checklist

Group raw channels from the same acquisition before creating partitions.

Fit scalers, references, selected features and TASP scores using training data only.

Tune hyperparameters and abstention thresholds on validation/calibration data only.

Reserve final held-out groups until the pipeline and selection rules are frozen.

Log random seeds, code commits, software versions and exact dataset identifiers.

Use consistent counting unit (acquisitions/cycles vs waveforms) when reporting sample sizes.

# Appendix B  Evidence inventory and dataset audit template

| Evidence claim | Source / required check | Status |
| --- | --- | --- |
| 12 transducers; frequency and temperature specification | Scientific Data 2019, Methods | Documented |
| Four reversible surface-disc positions | Scientific Data 2019, Methods | Documented |
| Two healthy cycles; one damaged cycle per location | Scientific Data 2019, Data Records | Documented |
| HDF5 fields and sampling-rate specifics | Inspect downloaded native HDF5 files | Not yet audited |
| Per-class independent sample counts | Acquisition manifest and grouping | Not yet audited |
| Actual ML performance and robustness gain | Run locked experiments E1–E6 | Not measured |
| Generalization beyond single laboratory specimen | Independent-specimen datasets needed | Not established |

# References and source record

[S1] SRMIST. Fundamentals of Composite Materials (21MEO101T), course syllabus, 2021 regulations, revised July 2024. User-supplied course PDF.

[R1] Moll, J., Kexel, C., Pötzsch, S., Rennoch, M., & Herrmann, A. S. (2019). Temperature affected guided wave propagation in a composite plate complementing the Open Guided Waves Platform. Scientific Data, 6, 191.
https://doi.org/10.1038/s41597-019-0208-1

[D1] Open Guided Waves, dataset #2, public download catalogue and source archive.
https://openguidedwaves.de/downloads/

[D2] Moll et al. (2019), dataset collection on figshare.
https://doi.org/10.6084/m9.figshare.c.4488089.v1

[R2] Moll, J., et al. (2019). Open Guided Waves - Online Platform for Ultrasonic Guided Wave Measurements. Structural Health Monitoring, 18(5-6), 1903-1914.
https://doi.org/10.1177/1475921718817169

[R3] Croxford, A. J., Moll, J., Wilcox, P. D., & Michaels, J. E. (2010). Efficient temperature compensation strategies for guided wave structural health monitoring. Ultrasonics, 50, 517-528.
https://doi.org/10.1016/j.ultras.2009.11.002

[R4] Reliability assessment of guided wave damage localization with deep learning uncertainty quantification methods (2024). NDT & E International, 144, 103099.
https://doi.org/10.1016/j.ndteint.2024.103099

[R5] Unsupervised temperature-compensated damage localization method based on damage to baseline autoencoder and delay-based probabilistic imaging (2025). Mechanical Systems and Signal Processing, 230, 112649.
https://doi.org/10.1016/j.ymssp.2025.112649

[D3] European Workshop on Structural Health Monitoring (2024), OGW Dataset #2 registry entry.
https://ewshm2024.com/frontend/index.php?page_id=381